CANcrusher Car Hack / Development Platform

Hardware/System Features:

- 3 independent CAN channels supporting DW-CAN, SW-CAN, and LSFT CAN based on the MCP2515 CAN controller.

- 1 LIN/KLINE channel

- Bluetooth radio (RN42) with built-in SPP (acts like a serial port)

- SIM808 GSM / GPS Radio

- USB

- SD Datalogging using Bill Greiman's SDfat library for up to 64GB cards

- Real time clock using the DS3231 high precision RTC

- Teensy 3.1 (ARM Cortex M4) running @ 96MHz

- 8 Multi-purpose Inputs and Outputs for directly interfacing with the vehicle (outputs are open collector negative outputs with optional pull-up resistors.)

- LED Indicators (4 LEDs)

SOFTWARE:

The firmware is Open Source (GNU General Public License), Arduino compatible and uses many of the libraries people know and love/hate.

On the computer-side, the initial software will be developed in Java with some of the following goals (some early versions of these features is done already):

- USB or Bluetooth communication with the CANcrusher

- CAN bus monitor allowing sending and receiving data, processing, reverse engineering, etc...

- Accepts CAN databases (*.dbc) format

- Imports and exports log files in Vector and Intrepid Controls Systems format

Early version of the binary file exporting tool:

COM Protocol for Bluetooth and USB interface:

The protocol is still in development, but here is an example of the data packets that will be exchanged between the CANcrusher and the computer when not in logging mode.

Early example of the CAN messages being sent to a Java GUI: